SIMATS ENGINEERING

ASSESSMENT TOOL 2

Case study

NAME: Siva Dakshitha K                                                                    REG.NO: 192425395

COURSE NAME: Computer Networks			COURSE CODE:CSA0703

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

Case study

QUESTIONS:4						                   Total Marks:40

1.A multinational IT company uses a cloud-based video conferencing platform for daily client meetings. During peak business hours, employees experience frequent voice distortion, frozen video frames, and delayed communication. Network monitoring reports indicate an 8% packet loss, 180 ms jitter, and fluctuating bandwidth utilization. The IT administrator suspects that the issue is related to the transport protocol or application-level buffering strategy.

### Questions

- Analyze whether the problem is primarily caused by the transport protocol (TCP/UDP) or the application layer.

- Justify your answer based on the communication requirements of real-time multimedia applications.

- Recommend suitable protocol-level or application-level improvements to enhance conferencing quality.

Answer

1. Introduction

Cloud-based video conferencing has become an essential communication technology in modern organizations. It allows employees, customers, and business partners from different geographical locations to communicate in real time through voice, video, screen sharing, and online collaboration. Applications such as Microsoft Teams, Zoom, Google Meet, and Cisco Webex are widely used because they improve productivity and reduce travel costs.

Real-time communication requires a network that provides low latency, low packet loss, low jitter, and stable bandwidth. Unlike file transfers or email services, video conferencing cannot tolerate long delays because audio and video must arrive continuously and in the correct sequence. Even a small interruption in packet delivery can reduce communication quality and negatively affect user experience.

In the given case study, the organization experiences the following network conditions:

- Packet Loss = 8%

- Jitter = 180 ms

- Fluctuating Bandwidth

These values indicate that the network is unable to maintain smooth communication during peak business hours. As a result, employees experience voice distortion, frozen video frames, and delayed conversations during client meetings. The IT administrator suspects that the problem may be related to either the transport protocol or the buffering mechanism used by the application. Therefore, a detailed analysis is required to identify the root cause and recommend suitable improvements.

2. Background Theory

To understand the problem, it is important to know how a video conferencing application works.

A typical cloud-based video conferencing system captures audio and video using the user's microphone and camera. The captured media is compressed using codecs and divided into small packets before transmission over the network. These packets travel through routers and switches until they reach the destination device, where they are reassembled and played back in real time.

The communication process consists of multiple networking layers.

- Application Layer: Handles video encoding, audio compression, buffering, adaptive bitrate control, packet recovery, and user interaction.

- Transport Layer: Responsible for delivering packets using either TCP or UDP.

- Network Layer: Routes packets across different networks using IP.

- Data Link Layer: Transfers frames between devices connected within the same local network.

- Physical Layer: Transmits data using cables, Wi-Fi, or optical fiber.

Among these layers, the Application Layer and Transport Layer play the most significant roles in determining conferencing quality.

3. Understanding the Network Parameters

3.1 Packet Loss

Packet loss occurs when one or more packets fail to reach their destination. This may happen because of network congestion, faulty hardware, overloaded routers, wireless interference, or insufficient bandwidth.

In this case, packet loss is 8%, which is considered very high for multimedia communication.

Effects of packet loss include:

- Missing words during conversations.

- Distorted audio.

- Frozen or skipped video frames.

- Poor synchronization between voice and video.

- Reduced meeting quality.

Normally, video conferencing applications work best when packet loss remains below 1%.

3.2 Jitter

Jitter refers to the variation in packet arrival time. Ideally, packets should arrive at equal intervals. However, because of network congestion and varying routing paths, packets may arrive too early or too late.

The measured jitter is 180 milliseconds, which is extremely high.

High jitter causes:

- Choppy audio.

- Delayed voice communication.

- Frozen video frames.

- Lip synchronization problems.

- Buffer overflows and underflows.

Most conferencing applications perform best when jitter is below 30 milliseconds.

3.3 Fluctuating Bandwidth

Bandwidth is the amount of data that can be transmitted over the network in one second.

When available bandwidth changes continuously, the application cannot maintain a constant video quality.

Effects include:

- Sudden reduction in video resolution.

- Frequent switching between HD and SD video.

- Increased buffering.

- Temporary freezing of video.

- Delay in screen sharing.

Stable bandwidth is essential for maintaining consistent conferencing quality.

4. Analysis of the Problem

The major question in this case study is whether the problem is caused mainly by the transport protocol or the application layer.

The analysis indicates that the primary cause is the Application Layer rather than the Transport Layer.

Video conferencing software generally uses UDP (User Datagram Protocol) because UDP allows continuous transmission without waiting for acknowledgements. Since live communication values speed over perfect reliability, UDP is the preferred transport protocol.

However, although UDP is appropriate, the application must effectively handle packet loss, jitter, and bandwidth fluctuations. This is achieved using adaptive bitrate streaming, jitter buffering, packet loss concealment, forward error correction, and intelligent media synchronization.

In the given case, the application is unable to compensate for the severe network variations. Consequently, users experience poor voice quality, frozen video, and communication delays.

Therefore, the transport protocol itself is not the main problem. Instead, the application layer fails to efficiently manage changing network conditions, resulting in degraded conferencing performance.

5. Justification Based on Real-Time Multimedia Communication Requirements

Real-time multimedia applications such as Microsoft Teams, Zoom, Google Meet, Cisco Webex, and Skype require continuous transmission of audio and video data. Unlike file transfers or emails, these applications cannot wait for retransmission of lost packets because even a small delay can interrupt live conversations.

The communication requirements of real-time multimedia applications are:

a) Low Latency

Latency is the total time taken for a packet to travel from the sender to the receiver.

For high-quality video conferencing:

- Less than 150 ms – Excellent communication

- 150–300 ms – Acceptable with slight delay

- Above 300 ms – Poor communication quality

The given network already experiences communication delays because packet loss and jitter increase overall latency.

b) Low Jitter

Jitter is the variation in packet arrival time.

If packets arrive irregularly:

- Voice becomes unclear.

- Video freezes.

- Audio and video become unsynchronized.

- Screen sharing becomes unstable.

Since the measured jitter is 180 ms, the conferencing application struggles to play media continuously.

c) Continuous Packet Delivery

Unlike downloading a file, video conferencing requires packets to arrive continuously.

Missing packets create:

- Silent audio gaps

- Frozen video

- Missing frames

- Interrupted conversations

Therefore, continuous packet delivery is more important than perfect reliability.

d) Stable Bandwidth

Video conferencing continuously adjusts video quality depending on available bandwidth.

If bandwidth changes frequently:

- HD video changes to SD.

- Video quality decreases.

- Additional buffering occurs.

- Screen sharing becomes slower.

Stable bandwidth improves conferencing quality significantly.

6. Why UDP is Preferred Instead of TCP

Video conferencing generally uses UDP (User Datagram Protocol) instead of TCP because speed is more important than guaranteed delivery.

UDP simply sends packets without waiting for acknowledgements.

Advantages include:

- Very low communication delay

- Faster packet transmission

- No connection establishment

- No retransmission delay

- Suitable for live communication

If one packet is lost, the application simply continues transmitting the next packet.

For example:

During a live meeting, losing one video frame is less noticeable than delaying the entire conversation.

7. Why TCP is Not Suitable

TCP provides reliable communication.

It guarantees:

- Error-free delivery

- Packet sequencing

- Acknowledgements

- Retransmissions

Although reliability is useful for downloading files, it becomes a disadvantage during live communication.

If one packet is lost:

- TCP waits.

- Sender retransmits the missing packet.

- Receiver waits before continuing.

This introduces additional delay.

As a result:

- Video freezes.

- Audio becomes delayed.

- Conversations become unnatural.

Therefore, TCP is generally avoided for live audio and video transmission.

8. Comparison Between TCP and UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection Type | Connection-Oriented | Connectionless |
| Reliability | High | Low |
| Speed | Slower | Faster |
| Acknowledgements | Required | Not Required |
| Retransmission | Yes | No |
| Latency | Higher | Lower |
| Packet Ordering | Guaranteed | Not Guaranteed |
| Suitable for Video Conferencing | No | Yes |
| Suitable for File Transfer | Yes | No |

From the comparison, UDP clearly satisfies the requirements of real-time multimedia communication.

9. RTP (Real-Time Transport Protocol)

Although UDP provides fast communication, it does not provide media synchronization.

Therefore, video conferencing applications use RTP (Real-Time Transport Protocol) over UDP.

RTP provides:

- Packet numbering

- Time stamps

- Audio-video synchronization

- Packet sequencing

- Better playback quality

Limitations

Although the proposed solution significantly improves conferencing quality, it also has certain limitations.

- QoS implementation requires compatible networking equipment.

- Adaptive bitrate may temporarily reduce video quality.

- Forward Error Correction increases bandwidth usage slightly.

- Dynamic jitter buffers require careful tuning.

- Additional infrastructure increases deployment cost.

- Continuous monitoring is required to maintain optimal performance.

However, these limitations are relatively minor compared to the improvements achieved in communication quality.

Conclusion

The analysis of the given case study indicates that the poor conferencing experience is primarily caused by the application layer rather than the transport protocol. The use of UDP is appropriate because real-time multimedia communication requires low latency and continuous packet transmission. Replacing UDP with TCP would increase delays due to acknowledgements and retransmissions, making communication less efficient.

The major issues arise because the application is unable to effectively manage 8% packet loss, 180 ms jitter, and fluctuating bandwidth. By implementing RTP over UDP, Quality of Service (QoS), Adaptive Bitrate Streaming, Dynamic Jitter Buffers, Forward Error Correction (FEC), and Packet Loss Concealment (PLC), the organization can significantly improve conferencing performance.

These enhancements reduce communication delay, minimize video freezing, improve voice clarity, and provide a reliable conferencing experience even during peak business hours. As a result, employees can communicate effectively with clients and colleagues, leading to improved productivity, better collaboration, and higher customer satisfaction.

2.A multinational software company hosts its web services across multiple continents. Employees frequently report long delays before the application homepage loads. Network analysis shows that the average DNS lookup time is approximately 800 ms, significantly affecting user experience. The organization plans to redesign its DNS infrastructure while ensuring continuous service availability during server failures.

### Questions

- Analyze the reasons behind the high DNS resolution time.

- Propose a suitable DNS architecture using techniques such as Anycast DNS, DNS pre-fetching, and TTL optimization to reduce the lookup time below 50 ms.

- Evaluate the advantages and limitations of your proposed architecture in terms of performance, scalability, and availability.

# Answer:

# Introduction

The Domain Name System (DNS) translates domain names into IP addresses before users can access websites or applications. A fast DNS response is essential for improving webpage loading speed and user experience. In this case, the DNS lookup time is 800 ms, which is much higher than the recommended response time, causing delays in accessing the application.

## Analysis of the Problem

The primary reason for the high DNS resolution time is an inefficient DNS infrastructure. Since the company serves users across multiple continents, relying on a limited number of DNS servers increases network latency and response time.

# Introduction

The Domain Name System (DNS) translates domain names into IP addresses before users can access websites or applications. A fast DNS response is essential for improving webpage loading speed and user experience. In this case, the DNS lookup time is 800 ms, which is much higher than the recommended response time, causing delays in accessing the application.

The main reasons are:

- DNS servers are located far from many users, increasing network delay.

- Heavy DNS traffic overloads the server during peak hours.

- Lack of DNS caching causes repeated queries to authoritative servers.

- Poor TTL configuration leads to frequent DNS lookups.

- Absence of load balancing results in uneven distribution of requests.

- No effective failover mechanism increases response time during server failures.

As a result, users experience slow homepage loading and reduced application performance.

## Proposed DNS Architecture

To reduce DNS lookup time below 50 ms, the organization should implement a distributed DNS architecture with the following techniques:

### 1. Anycast DNS

- Deploy multiple DNS servers in different geographic regions.

- Use the same IP address for all DNS servers.

- Automatically route user requests to the nearest DNS server.

### 2. DNS Pre-fetching

- Resolve frequently accessed domain names before users request them.

- Reduce waiting time for repeated website visits.

### 3. TTL Optimization

- Configure appropriate TTL values.

- Frequently changing records can use lower TTL values.

- Stable records can use higher TTL values to improve caching efficiency.

### 4. DNS Caching

- Store recently resolved DNS records in recursive resolvers and client systems.

- Reduce repeated queries to authoritative DNS servers.

### 5. Secondary DNS Servers

- Deploy backup DNS servers to provide continuous service if the primary server fails.

### Proposed Architecture Flow

User Request

↓

Nearest Anycast DNS Server

↓

Recursive DNS Cache

↓

Authoritative DNS Server (if required)

↓

Return IP Address

↓

Application Server

This architecture minimizes lookup latency while maintaining high availability.

## Evaluation of the Proposed Architecture

### Performance

- DNS lookup time is reduced from 800 ms to below 50 ms.

- Faster website loading.

- Reduced network latency.

- Improved user experience.

### Scalability

- Additional DNS servers can be deployed easily as the number of users increases.

- Load is distributed across multiple servers.

- Suitable for global organizations.

### Availability

- Multiple DNS servers eliminate a single point of failure.

- Automatic failover ensures uninterrupted service.

- Anycast routing redirects users to healthy DNS servers if one server becomes unavailable.

### Advantages

- Faster DNS resolution.

- High availability and reliability.

- Better load balancing.

- Improved scalability.

- Reduced server workload through DNS caching.

- Enhanced user experience.

### Limitations

- Higher deployment and maintenance costs.

- More complex network configuration.

- TTL values must be carefully managed to balance performance and record updates.

- Global DNS infrastructure requires continuous monitoring.

## Conclusion

The high DNS resolution time is mainly caused by a centralized and inefficient DNS infrastructure. Implementing a distributed DNS architecture using Anycast DNS, DNS pre-fetching, TTL optimization, DNS caching, and secondary DNS servers significantly reduces lookup time, improves application performance, and ensures high availability during server failures. This solution provides a scalable, reliable, and efficient DNS system capable of supporting global users with minimal latency.

3.A cloud service provider delivers applications to millions of users worldwide. During promotional events, users experience intermittent delays in accessing cloud-hosted services. Investigation reveals that DNS queries are taking nearly 800 ms because all requests are directed to a single primary DNS server. The company wants a highly available DNS solution capable of handling global traffic efficiently.

### Questions

- Examine the impact of centralized DNS architecture on application performance.

- Design a distributed DNS solution that minimizes lookup latency while maintaining high availability.

- Analyze how TTL values, Anycast routing, and DNS caching influence system performance and fault tolerance.

# Answer:

## Introduction

The Domain Name System (DNS) is responsible for translating domain names into IP addresses before users can access cloud applications. A well-designed DNS infrastructure is essential for fast response time, high availability, and reliable access to services. In this case, all DNS requests are directed to a single primary DNS server, resulting in an average lookup time of 800 ms and poor user experience during high-traffic events.

## Impact of Centralized DNS Architecture

A centralized DNS architecture negatively affects application performance because all user requests are processed by one DNS server.

The major impacts are:

- High DNS lookup latency due to server overload.

- Single Point of Failure (SPOF)—if the primary DNS server fails, users cannot resolve domain names.

- Increased network congestion during promotional events.

- Longer webpage loading time, reducing user satisfaction.

- Limited scalability, as one server cannot efficiently handle millions of DNS requests.

- Higher response time for users located far from the DNS server.

As a result, cloud-hosted applications become slow and less reliable during peak traffic periods.

## Proposed Distributed DNS Solution

To improve performance and availability, the organization should implement a distributed DNS architecture.

### 1. Anycast DNS

- Deploy DNS servers in multiple geographic regions.

- Assign the same IP address to all DNS servers.

- Automatically route users to the nearest available DNS server.

### 2. DNS Caching

- Store recently resolved DNS records in recursive resolvers and client systems.

- Reduce repeated requests to authoritative DNS servers.

- Improve response speed.

### 3. Optimized TTL Values

- Use higher TTL values for stable records to improve cache efficiency.

- Use lower TTL values for frequently changing records to ensure updated information.

### 4. Secondary DNS Servers

- Deploy backup DNS servers for redundancy.

- Provide automatic failover if the primary server becomes unavailable.

### 5. Load Balancing

- Evenly distribute DNS queries among multiple servers.

- Prevent server overload during high-traffic events.

### Architecture Flow

User Request

↓

Nearest Anycast DNS Server

↓

Recursive DNS Cache

↓

Authoritative DNS Server (if required)

↓

Cloud Application Server

This architecture significantly reduces DNS lookup time while ensuring continuous service availability. In the given case, the company uses TCP for communication between trading terminals and the central server. During heavy market activity, TCP retransmissions increase communication latency, resulting in delayed order execution. Therefore, the communication system must achieve both high speed and reliable data delivery.

A well-designed DNS infrastructure is essential for fast response time, high availability, and reliable access to services. In this case, all DNS requests are directed to a single primary DNS server, resulting in an average lookup time of 800 ms and poor user experience during high-traffic events.

## Influence of TTL, Anycast Routing, and DNS Caching

### TTL (Time to Live)

- Determines how long DNS records remain in cache.

- Higher TTL reduces repeated DNS queries and lowers server load.

- Lower TTL allows quicker updates but increases DNS traffic.

### Anycast Routing

- Directs users to the geographically nearest DNS server.

- Reduces network latency.

- Improves response time.

- Automatically redirects traffic if a server fails, increasing fault tolerance.

### DNS Caching

- Stores previously resolved DNS records.

- Reduces lookup time for repeated requests.

- Decreases traffic to authoritative DNS servers.

- Improves overall system performance and scalability.

## Advantages of the Proposed Solution

- Faster DNS resolution (below 50 ms).

- High availability with no single point of failure.

- Better scalability for millions of users.

- Reduced server workload.

- Improved load balancing.

- Enhanced fault tolerance.

- Better user experience during peak traffic.

## Limitations

- Higher implementation and maintenance cost.

- More complex network configuration.

- Requires continuous monitoring and synchronization.

- Incorrect TTL settings may affect update speed.

## Conclusion

The delays are mainly caused by the centralized DNS architecture, where all DNS requests are handled by a single server. A distributed DNS solution using Anycast routing, DNS caching, optimized TTL values, load balancing, and secondary DNS servers greatly improves performance, scalability, and availability.A well-designed DNS infrastructure is essential for fast response time, high availability, and reliable access to services. In this case, all DNS requests are directed to a single primary DNS server, resulting in an average lookup time of 800 ms and poor user experience during high-traffic events.

This architecture minimizes DNS lookup latency, provides fault tolerance, and ensures reliable access to cloud-hosted services even during heavy promotional traffic.

4.A financial institution operates an electronic stock trading platform where thousands of buy and sell orders are submitted every second. During market opening hours, the average order acknowledgment latency increases from 1 ms to 40 ms, resulting in delayed trade confirmations and customer dissatisfaction. Network engineers observe increased TCP retransmissions, longer connection establishment times, and excessive buffering.

### Questions

- Analyze three possible TCP-related factors responsible for the latency increase, considering flow control, Nagle's algorithm, and SYN queue limitations.

- Explain how each factor affects transaction processing in high-frequency systems

- Recommend appropriate TCP configuration changes to reduce latency and improve transaction reliability.

# Answer:

## Introduction

High-frequency trading (HFT) systems require extremely fast communication because even a few milliseconds of delay can affect stock prices and trading profits. In the given case, the company uses TCP for communication between trading terminals and the central server. During heavy market activity, TCP retransmissions increase communication latency, resulting in delayed order execution. Therefore, the communication system must achieve both high speed and reliable data delivery.

## Analysis of the Problem

TCP is a connection-oriented and reliable transport protocol. In the given case, the company uses TCP for communication between trading terminals and the central server. During heavy market activity, TCP retransmissions increase communication latency, resulting in delayed order execution. Therefore, the communication system must achieve both high speed and reliable data delivery.

It guarantees accurate data delivery through acknowledgements, sequencing, flow control, and retransmission of lost packets.

During heavy trading periods:

- Network congestion increases.

- Some packets are delayed or lost.

- TCP retransmits lost packets before delivering data.

- The sender waits for acknowledgements before continuing communication.

- Congestion control mechanisms reduce transmission speed.

These features ensure reliability but introduce additional delay. In the given case, the company uses TCP for communication between trading terminals and the central server. During heavy market activity, TCP retransmissions increase communication latency, resulting in delayed order execution. Therefore, the communication system must achieve both high speed and reliable data delivery.

In a high-frequency trading environment, even small delays can result in slower order execution and financial loss. Therefore, TCP retransmissions are the primary reason for increased latency.

## Recommended Communication Strategy

A balanced communication strategy should combine both speed and reliability.

### 1. Use UDP for Time-Critical Market Data

- Use UDP to transmit live stock prices and market updates.

- UDP provides low latency because it does not wait for acknowledgements or retransmissions.

### 2. Use TCP for Critical Transactions

- Continue using TCP for trade confirmations, account updates, and final transaction records.

- TCP ensures reliable and accurate delivery of important financial information.

### 3. Implement Application-Level Reliability

- Add sequence numbers and timestamps to detect missing or delayed packets.

- Use application-level error detection and recovery where necessary.

### 4. Prioritize Network Traffic

- Implement Quality of Service (QoS) to prioritize trading data over less important traffic.

- Reduce network congestion during peak trading hours.

### 5. High-Speed Network Infrastructure

- Use low-latency switches, dedicated communication links, and high-performance servers.

- Optimize routing paths between trading terminals and servers.

This hybrid communication strategy provides both fast response time and reliable transaction processing. In the given case, the company uses TCP for communication between trading terminals and the central server. During heavy market activity, TCP retransmissions increase communication latency, resulting in delayed order execution. Therefore, the communication system must achieve both high speed and reliable data delivery.

## Evaluation of the Proposed Solution

### Latency

- UDP minimizes communication delay for real-time market data.

- QoS and optimized infrastructure further reduce transmission latency.

- Faster order execution improves trading performance.

### Reliability

- TCP guarantees reliable delivery of critical financial transactions.

- Application-level validation ensures data integrity.

- Important trading records remain accurate and secure.

### Implementation Complexity

- Implementing both TCP and UDP increases system complexity.

- Additional monitoring and synchronization mechanisms are required.

- Higher deployment and maintenance costs are involved.

- Skilled network administrators are needed to manage the hybrid architectureAdvantages

- Faster order execution.

- Reduced communication latency.

- Reliable delivery of critical financial transactions.

- Better network utilization.

- Improved scalability during heavy trading activity.

- Higher overall trading efficiency.

## Limitations

- Hybrid architecture is more complex to implement.

- Increased infrastructure and maintenance costs.

- Requires careful synchronization between TCP and UDP communication.

- Additional security and monitoring mechanisms are necessary.

## Conclusion

TCP retransmissions increase latency because they require acknowledgements, retransmissions, and congestion control before delivering data. In the given case, the company uses TCP for communication between trading terminals and the central server. During heavy market activity, TCP retransmissions increase communication latency, resulting in delayed order execution. Therefore, the communication system must achieve both high speed and reliable data delivery.A hybrid communication strategy, where UDP is used for real-time market data and TCP is used for critical financial transactions, provides the best balance between speed and reliability. Combined with Quality of Service (QoS), application-level error handling, and high-speed network infrastructure, this solution significantly improves trading performance while maintaining secure and reliable transaction processing.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understanding; some issues missed | Poor understanding; problem not identified correctly |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |
| Clarity | 10 | Clear, well-structured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |