SIMATS ENGINEERING

ASSESSMENT TOOL 1

NAME: Siva Dakshitha K					   REG.NO :192425395

COURSE NAME: COMPUTER NETWORKS                 COURSE CODE: CSA0703

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

ANS:

Subnetting of 192.168.20.0/24

Problem Understanding

Given network: 192.168.20.0/24

Required subnet sizes:

- Lab A → /26

- Lab B → /27

- Lab C → /28

Determine:

- Network Address

- Host Range

- Broadcast Address

- Usable Hosts

Method Selection

Formula:

- Total addresses = 232-prefix

- Usable hosts = Total addresses − 2

Subnet sizes:

| Prefix | Addresses | Usable Hosts |
| --- | --- | --- |
| /26 | 64 | 62 |
| /27 | 32 | 30 |
| /28 | 16 | 14 |

Solution Process

Lab A (/26)

Network Address:

192.168.20.0/26

Host Range:

192.168.20.1 – 192.168.20.62

Broadcast:

192.168.20.63

Usable Hosts:

64 − 2 = 62

Lab B (/27)

Next available subnet starts after .63

Network:

192.168.20.64/27

Host Range:

192.168.20.65 – 192.168.20.94

Broadcast:

192.168.20.95

Usable Hosts:

32 − 2 = 30

Lab C (/28)

Next subnet:

Network:

192.168.20.96/28

Host Range:

192.168.20.97 – 192.168.20.110

Broadcast:

192.168.20.111

Usable Hosts:

16 − 2 = 14

Final Answer

| Lab | Network | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- |
| A | 192.168.20.0/26 | 192.168.20.1–62 | 192.168.20.63 | 62 |
| B | 192.168.20.64/27 | 192.168.20.65–94 | 192.168.20.95 | 30 |
| C | 192.168.20.96/28 | 192.168.20.97–110 | 192.168.20.111 | 14 |

Interpretation

Each lab receives an independent subnet with its own network and broadcast addresses, ensuring efficient IP utilization and reduced broadcast traffic.

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

ANS:

Subnet Analysis

Given

System 1

192.168.1.130/26

System 2

192.168.1.190/26

Subnet mask

255.255.255.192

Problem Understanding

Need to determine

- Network

- Broadcast

- Host Range

- Verify IP belongs

- Usable Hosts

Method Selection

/26 block size

256−192=64

Subnets

0

64

128

192

Solution Process

System 1

IP

192.168.1.130

Falls in

128–191

Network

192.168.1.128

Broadcast

192.168.1.191

Host Range

192.168.1.129–190

130 lies inside the range ✔

Usable Hosts

64−2=62

System 2

IP

192.168.1.190

Also lies in

128–191

Network

192.168.1.128

Broadcast

192.168.1.191

Host Range

129–190

190 belongs ✔

Usable Hosts

62

Final Answer

| System | Network | Host Range | Broadcast | Valid? | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| 1 | 192.168.1.128 | 129–190 | 191 | Yes | 62 |
| 2 | 192.168.1.128 | 129–190 | 191 | Yes | 62 |

Interpretation

Both systems belong to the same subnet, allowing direct communication without routing.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

ANS:

Subnet Allocation

Given

172.16.0.0/16

Departments

- HR /25

- Sales /26

- IT /27

- Admin /28

Method Selection

Allocate sequentially.

Solution Process

HR

Network

172.16.0.0/25

Hosts

172.16.0.1–126

Broadcast

172.16.0.127

Usable

126

Sales

172.16.0.128/26

Hosts

172.16.0.129–190

Broadcast

172.16.0.191

Usable

62

IT

172.16.0.192/27

Hosts

172.16.0.193–222

Broadcast

172.16.0.223

Usable

30

Admin

172.16.0.224/28

Hosts

172.16.0.225–238

Broadcast

172.16.0.239

Usable

14

Remaining Unused Range

Allocated until

172.16.0.239

Remaining

172.16.0.240 – 172.16.255.255

Final Answer

| Department | Network | Host Range | Broadcast | Usable |
| --- | --- | --- | --- | --- |
| HR | 172.16.0.0/25 | 1–126 | 127 | 126 |
| Sales | 172.16.0.128/26 | 129–190 | 191 | 62 |
| IT | 172.16.0.192/27 | 193–222 | 223 | 30 |
| Admin | 172.16.0.224/28 | 225–238 | 239 | 14 |

Remaining:

172.16.0.240 – 172.16.255.255

Interpretation

Variable subnet sizes efficiently allocate IP addresses while leaving a large unused address space for future expansion.

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

ANS:

/27 Subnetting

Given

172.16.10.0/24

Mask

/27

Method Selection

Borrowed bits

27−24=3

Subnets

2³=8

Hosts

2⁵−2=30

Solution Process

Subnets

0

32

64

96

128

160

192

224

IP

172.16.10.78

Falls inside

64–95

Network

172.16.10.64

Broadcast

172.16.10.95

Host Range

65–94

Now check

172.16.10.95

This is

Broadcast Address

Hence

Not a valid host.

Final Answer

| Item | Answer |
| --- | --- |
| Number of Subnets | 8 |
| Usable Hosts/Subnet | 30 |
| Network | 172.16.10.64 |
| Broadcast | 172.16.10.95 |
| Host Range | 172.16.10.65–94 |
| 172.16.10.95 Same Subnet? | Yes (Broadcast only, not usable host) |

Interpretation

The IP 172.16.10.78 is a valid host within the subnet 172.16.10.64/27, while 172.16.10.95 is the broadcast address and cannot be assigned to a device.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000: ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

ANS:

IPv6 Addressing

Given

2001:0db8:0000:0000:0000: ff00:0042:8329

Problem Understanding

Need

- Compress

- Expand

- Network Prefix

- Host Count

Method Selection

IPv6 Rules

- Remove leading zeros.

- Replace the longest sequence of consecutive zero groups with "::" (once only).

- For a /64 prefix, the first 64 bits form the network prefix.

Solution Process

Compressed

2001:db8::ff00:42:8329

Expanded

2001:0db8:0000:0000:0000:ff00:0042:8329

Network Prefix (/64)

2001:0db8:0000:0000::/64

Number of Hosts

Host bits

128−64=64

Hosts

2⁶⁴

=

18,446,744,073,709,551,616

Final Answer

| Item | Result |
| --- | --- |
| Compressed | 2001:db8::ff00:42:8329 |
| Expanded | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Network Prefix | 2001:0db8:0000:0000::/64 |
| Host Addresses | 2⁶⁴ = 18,446,744,073,709,551,616 |

Interpretation

IPv6 provides an extremely large address space. A /64 subnet supports 2⁶⁴ interface identifiers, making it suitable for large-scale and future-proof network deployments.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |