SIMATS ENGINEERING

ASSESSMENT TOOL 2

NAME: Siva Dakshitha K                                            REG.NO:192425395

COURSE NAME: COMPUTER NETWORKS           COURSE CODE: CSA0703

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid host IP address range for each subnet.

- Calculate the number of usable host addresses available in each subnet.

- Determine the remaining unused IP address range within the given network block.

ANS:

Smart Campus Network Subnetting

Given:

- Network Address: 192.168.100.0/24

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

Step 1: School of Computing (/25)

Subnet Mask:

255.255.255.128

Block Size:

128 addresses

Network Address:

192.168.100.0

Broadcast Address:

192.168.100.127

Host Range:

192.168.100.1 – 192.168.100.126

Usable Hosts:

2^(32−25) − 2

= 2⁷ − 2

= 128 − 2

= 126

Step 2: School of Electronics (/26)

Subnet Mask:

255.255.255.192

Block Size:

64 addresses

Next available network begins at 192.168.100.128

Network Address:

192.168.100.128

Broadcast Address:

192.168.100.191

Host Range:

192.168.100.129 – 192.168.100.190

Usable Hosts:

2⁶−2

=64−2

= 62

Step 3: School of Mechanical Engineering (/27)

Subnet Mask:

255.255.255.224

Block Size:

32 addresses

Next available network:

192.168.100.192

Broadcast Address:

192.168.100.223

Host Range:

192.168.100.193 – 192.168.100.222

Usable Hosts:

2⁵−2

=32−2

= 30

Step 4: Remaining Unused Address Space

Remaining addresses:

192.168.100.224

↓

192.168.100.255

Unused subnet:

192.168.100.224/27

Total unused addresses:

32

Final Answer

| Department | Network | Broadcast | Host Range | Hosts |
| --- | --- | --- | --- | --- |
| Computing | 192.168.100.0 | 192.168.100.127 | 192.168.100.1–126 | 126 |
| Electronics | 192.168.100.128 | 192.168.100.191 | 192.168.100.129–190 | 62 |
| Mechanical | 192.168.100.192 | 192.168.100.223 | 192.168.100.193–222 | 30 |

Unused Range:

192.168.100.224–192.168.100.255

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid range of host IP addresses.

- Calculate the number of usable hosts in each subnet.

- Identify the unused IP address range remaining within the allocated IP block.

ANS:

Company Network Design

Given

Network:

10.10.0.0/16

Departments:

- Software Development /24

- QA /25

- HR /26

- Executive /27

Software Development

Mask:

255.255.255.0

Network:

10.10.0.0

Broadcast:

10.10.0.255

Host Range:

10.10.0.1–10.10.0.254

Hosts:

2⁸−2

=254

Quality Assurance

Mask:

255.255.255.128

Next Network:

10.10.1.0

Broadcast:

10.10.1.127

Host Range:

10.10.1.1–126

Hosts:

126

Human Resources

Network:

10.10.1.128

Broadcast:

10.10.1.191

Host Range:

10.10.1.129–190

Hosts:

62

Executive

Network:

10.10.1.192

Broadcast:

10.10.1.223

Host Range:

10.10.1.193–222

Hosts:

30

Remaining Address Space

Unused inside allocated section:

10.10.1.224–10.10.1.255

Remaining entire /16 block:

10.10.2.0–10.10.255.255

Final Table

| Department | Network | Broadcast | Host Range | Hosts |
| --- | --- | --- | --- | --- |
| Software | 10.10.0.0 | 10.10.0.255 | 10.10.0.1–254 | 254 |
| QA | 10.10.1.0 | 10.10.1.127 | 10.10.1.1–126 | 126 |
| HR | 10.10.1.128 | 10.10.1.191 | 10.10.1.129–190 | 62 |
| Executive | 10.10.1.192 | 10.10.1.223 | 10.10.1.193–222 | 30 |

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- the total number of possible host addresses available within this network.

ANS:

IPv6 Migration

Given

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Step 1

Compressed Form

2001:db8:abcd::/64

Step 2

Expanded Form

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Step 3

Network Prefix

Since prefix = /64

Network:

2001:db8:abcd:0000::/64

Host Portion:

0000:0000:0000:0000

Step 4

Host Bits

128−64

=64 bits

Hosts

2⁶⁴

=18,446,744,073,709,551,616

Step 5

Total Possible Hosts

18,446,744,073,709,551,616

(approximately 18.4 quintillion addresses)

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

# ANS:

# Routing

### Routing Table

Network

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

Destination:

192.168.2.45

## Binary Verification

192.168.2.45

Subnet Mask:

255.255.255.0

Network ID

192.168.2.45

AND

255.255.255.0

=

192.168.2.0

Therefore,

Destination subnet:

192.168.2.0/24

Matching Entry:

192.168.2.0/24

Next Hop:

Router interface connected to

192.168.2.0/24

Default Route

0.0.0.0/0

Used whenever no route matches.

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

# ANS:

# Office LAN Design

Given

Administration

192.168.10.0/26

Finance

192.168.10.64/26

## Administration LAN

Subnet Mask

255.255.255.192

Block Size

64

Network

192.168.10.0

Broadcast

192.168.10.63

Host Range

192.168.10.1–62

Gateway

192.168.10.1

Suggested PCs

PC1

192.168.10.2

PC2

192.168.10.3

PC3

192.168.10.4

## Finance LAN

Network

192.168.10.64

Broadcast

192.168.10.127

Host Range

192.168.10.65–126

Gateway

192.168.10.65

Suggested PCs

PC1

192.168.10.66

PC2

192.168.10.67

PC3

192.168.10.68

## Final Summary Table

| LAN | Network Address | Broadcast Address | Valid Host Range | Default Gateway | Sample Host IPs |
| --- | --- | --- | --- | --- | --- |
| Administration | 192.168.10.0/26 | 192.168.10.63 | 192.168.10.1–192.168.10.62 | 192.168.10.1 | 192.168.10.2, 192.168.10.3, 192.168.10.4 |
| Finance | 192.168.10.64/26 | 192.168.10.127 | 192.168.10.65–192.168.10.126 | 192.168.10.65 | 192.168.10.66, 192.168.10.67, 192.168.10.68 |

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |