SIMATS ENGINEERING

ASSESSMENT TOOL 3

NAME: Siva Dakshitha K                                                           REG.NO:192425395

COURSE NAME: COMPUTER NETWORKS                        COURSE CODE: CSA0703

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software:Cisco Packet Tracer,Wireshark

Annexure A

Debugging & Optimisation Task

1. A network administrator has configured a workstation with the following settings:

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

- Identifying the subnet to which the configured IP address belongs.

- Determining the network address, broadcast address, and valid host range.

- Verifying whether the configured IP address and gateway are valid.

- Identifying the configuration errors.

- Optimizing the configuration by assigning an appropriate IP address and default gateway.

- Calculating the total number of usable host addresses after correction.

ANS:

IPv4 Network Configuration Debugging

Given

- IP Address = 192.168.10.65

- Subnet Mask = 255.255.255.192 (/26)

- Default Gateway = 192.168.10.1

Step 1: Identify the Subnet

A /26 subnet has a block size of 64. Subnets are:

| Subnet | Network Address | Host Range | Broadcast |
| --- | --- | --- | --- |
| 1 | 192.168.10.0/26 | 1–62 | 63 |
| 2 | 192.168.10.64/26 | 65–126 | 127 |
| 3 | 192.168.10.128/26 | 129–190 | 191 |
| 4 | 192.168.10.192/26 | 193–254 | 255 |

The IP 192.168.10.65 belongs to

Subnet: 192.168.10.64/26

Step 2: Network Details

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Valid Host Range: 192.168.10.65 – 192.168.10.126

Step 3: Verify IP Address

Configured IP

192.168.10.65

Host range

65–126

✅ Valid

Step 4: Verify Gateway

Configured Gateway

192.168.10.1

Gateway belongs to subnet

192.168.10.0/26

Workstation belongs to

192.168.10.64/26

❌ Gateway is in a different subnet.

Step 5: Configuration Error

- IP Address → Correct

- Gateway → Incorrect (different subnet)

Step 6: Optimized Configuration

Example:

- IP Address = 192.168.10.66

- Subnet Mask = 255.255.255.192

- Default Gateway = 192.168.10.65

(or any valid gateway between 65–126)

Step 7: Number of Usable Hosts

For /26

26=64

Usable hosts

64-2=62
Answer: 62 usable hosts

2. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

### Task

Debug the IPv6 configuration by:

- Expanding both IPv6 addresses into their complete notation.

- Determining the network prefix of each device.

- Verifying whether both devices belong to the same subnet.

- Identifying the addressing or prefix mismatch.

- Optimizing the IPv6 configuration by assigning appropriate addresses.

- Calculating the number of available host addresses within the corrected subnet.

ANS:

IPv6 Debugging

Given

Device A

2001:db8::ff00:42:8329/64

Device B

2001:db8:0:0:1::1/64

Step 1: Expand Addresses

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Network Prefix

/64 means first four blocks.

Device A

2001:db8:0000:0000::/64

Device B

2001:db8:0000:0000::/64

Step 3: Same Subnet?

Yes.

Both have

2001:db8:0:0::/64

✅ Same subnet.

Step 4: Addressing Error

No prefix mismatch.

Likely issues:

- Wrong gateway

- Duplicate address

- Firewall

- Interface down

- Neighbor Discovery failure

Step 5: Optimized Addressing

Device A

2001:db8:0:0::10/64

Device B

2001:db8:0:0::20/64

Gateway

2001:db8:0:0::1

Step 6: Number of Hosts

Remaining bits

128−64=64

Hosts

264=18,446,744,073,709,551,616 addresses

3. An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

- Identifying the network address, broadcast address, and host range for each subnet.

- Calculating the number of usable and unused IP addresses.

- Identifying inefficient subnet allocations.

- Debugging the addressing plan by locating subnet allocation mistakes.

- Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

ANS:

Subnet Optimization (VLSM)

Network

192.168.1.0/24

Existing Allocation

Department A (/26)

Network

192.168.1.0

Broadcast

192.168.1.63

Hosts

1–62

Usable

62

Department B (/27)

Network

192.168.1.64

Broadcast

192.168.1.95

Hosts

65–94

Usable

30

Department C (/28)

Network

192.168.1.96

Broadcast

192.168.1.111

Hosts

97–110

Usable

14

Unused Addresses

Remaining

192.168.1.112–255

Total

144 addresses unused.

Inefficiency

- One department has unused addresses.

- Another department lacks sufficient hosts.

- Address space is wasted.

Optimized VLSM Example

| Department | Hosts Needed | Subnet |
| --- | --- | --- |
| Dept A | 50 | /26 |
| Dept B | 25 | /27 |
| Dept C | 12 | /28 |
| Future Dept | 12 | /28 |
| Future Dept | 6 | /29 |

Advantages

- Better utilization

- Less wastage

- Supports future growth

- Flexible expansion

4. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

- Expanding both IPv6 addresses into their complete notation.

- Determining the network prefix of each device.

- Verifying whether both devices belong to the same subnet.

- Identifying the addressing or prefix mismatch.

- Optimizing the IPv6 configuration by assigning appropriate addresses.

- Calculating the number of available host addresses within the corrected subnet.

ANS:

IPv6 Debugging

(Same as Question 2)

Expanded Addresses

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Network Prefix

2001:db8:0:0::/64

Same subnet

✅ Yes

Optimized addresses

Device A → 2001:db8:0:0::10/64

Device B → 2001:db8:0:0::20/64

Gateway → 2001:db8:0:0::1

Hosts

264=18,446,744,073,709,551,616

5. A router contains the following routing table.

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Task

Debug the routing configuration by:

- Identifying the matching routing entry using the longest-prefix match.

- Determining the next-hop router selected for packet forwarding.

- Verifying whether the destination IP belongs to the intended subnet.

- Identifying any routing configuration errors.

- Optimizing the routing table to improve packet forwarding efficiency.

- Identifying the route that will be used if no matching destination exists.

ANS:

Routing Table Debugging

Routing Table

| Destination | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default | 10.0.0.1 |

Destination packet

192.168.2.100

Step 1: Longest Prefix Match

Destination

192.168.2.100

Matches

192.168.2.0/24

Step 2: Next-Hop Router

10.0.0.3

Step 3: Verify Destination Subnet

192.168.2.100 belongs to

192.168.2.0/24

✅ Correct subnet.

Step 4: Possible Errors

Possible reasons packets fail:

- Incorrect next-hop IP

- Next-hop router unreachable

- Interface shutdown

- Missing return route

- Incorrect subnet mask

- Routing protocol not updated

Step 5: Optimized Routing Table

| Destination | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| 192.168.3.0/24 | 10.0.0.4 (if required) |
| Default | 10.0.0.1 |

This improves scalability while keeping the default route for unknown destinations.

Step 6: Default Route

If no destination matches,

Packet is forwarded to

10.0.0.1 (Default Gateway)

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |