SIMATS ENGINEERING – C01(Assessment Tool 1)

Name:  Siva Dakshitha K                                        Reg.No: 192425395

Course Code: CSA0703                                         Course: Computer Networks

1. File Transfer using Transport and Data Link Layer

Given:

- File size = 150 MB = 150 × 1,000,000 = 150,000,000 bytes

- Segment size = 1500 bytes

- Data Link overhead/frame = 40 bytes

- Bandwidth = 50 Mbps

Calculation

Number of segments

150,000,0001500=100,000 segments

Number of frames

= 100,000 frames

Total overhead

100,000×40=4,000,000 bytes=4 MB

Total data transmitted

150+4=154 MB

Convert to bits:

154×8=1232 Mb

Transmission time

123250=24.64 seconds

Answer

- Total segments = 100,000

- Total frames = 100,000

- Data Link overhead = 4 MB

- Transmission time = 24.64 seconds

Explanation

The Transport Layer divides the file into segments and provides reliable delivery using acknowledgements, sequencing, and error recovery. The Data Link Layer creates frames, detects transmission errors using CRC, and retransmits damaged frames, ensuring reliable communication between directly connected devices.

2. Sliding Window Protocol

Given

- Window size = 6

- Total frames = 48

Calculation

Transmission windows

48÷6=8

One frame is lost in every window.

Retransmissions

8×1=8

Answer

- Number of windows = 8

- Total retransmissions = 8 frames

Explanation

Flow control prevents the sender from transmitting more frames than the receiver can process. This reduces congestion, avoids buffer overflow, and improves communication efficiency.

3. IP Fragmentation

Given

- Packet size = 12,000 bytes

- MTU = 1500 bytes

- Header = 20 bytes

Payload per fragment

1500-20=1480 bytes

Number of fragments

12000/1480=9

Header overhead

9×20=180 bytes

Answer

- Fragments created = 9

- Header overhead = 180 bytes

Explanation

The Network Layer divides large packets into fragments. Each fragment carries its own IP header. At the destination, the fragments are reassembled using fragment offset and identification fields to recover the original packet.

4. Sliding Window Protocol

Answer

Since the values are the same as Question 2:

- Number of windows = 8

- Retransmissions = 8 frames

Explanation

Flow control regulates data transmission according to the receiver's capacity, preventing data loss and improving network efficiency.

5. Session Layer Synchronization

Given

- File size = 600 MB

- Checkpoint every 60 MB

- Failure after 390 MB

Calculation

Checkpoints before failure

390÷60=6

Last checkpoint

6×60=360 MB

Retransmitted data

390-360=30 MB

Answer

- Checkpoints created = 6

- Data retransmitted = 30 MB

Explanation

The Session Layer inserts synchronization checkpoints. If communication fails, transmission resumes from the last checkpoint instead of restarting the entire transfer, saving time and bandwidth.

6. Presentation Layer Compression

Given

Original size = 900 MB

Compression = 40%

Encryption overhead = 5%

Calculation

Compressed size

900×0.60=540 MB

Encrypted size

540×1.05=567 MB

Reduction

900-567=333 MB

Percentage reduction

333900×100=37%

Answer

- Compressed size = 540 MB

- Encrypted size = 567 MB

- Total reduction = 37%

Explanation

The Presentation Layer compresses data to reduce transmission time and encrypts it to maintain confidentiality and data security during communication.

7. FTP File Transfer

Given

- File size = 3 GB = 3000 MB

- Request size = 3 MB

- Throughput = 120 Mbps

Calculation

Requests

3000÷3=1000

Convert to megabits

3000×8=24000 Mb

Transmission time

24000÷120=200 seconds

Answer

- Requests = 1000

- Transmission time = 200 seconds

Explanation

The Application Layer provides services such as FTP, user authentication, file management, and reliable communication between applications.

8. End-to-End Delay

Calculation

Processing delay

4+3+5+2+1=15 ms

Total delay

15+18+12=45 ms

Answer

- Total end-to-end delay = 45 ms

Explanation

- Application Layer prepares user data.

- Transport Layer segments data.

- Network Layer determines routing.

- Data Link Layer frames the data.

- Physical Layer transmits bits.

- Propagation delay is the travel time through the medium.

- Transmission delay is the time required to place data onto the link.

9. Frame Overhead

Given

Useful data = 80 MB

Frame size = 1600 bytes

Overhead = 80 bytes

Useful data/frame

1600-80=1520 bytes

Calculation

Frames

80,000,000÷1520≈52,632

Total overhead

52,632×80=4,210,560 bytes

Efficiency

15201600×100=95%

Answer

- Frames required = 52,632

- Total overhead = 4,210,560 bytes

- Transmission efficiency = 95%

Explanation

Protocol overhead consumes bandwidth because headers and trailers are transmitted along with useful data. Higher overhead reduces overall transmission efficiency.

10. Healthcare Network

Given

Original file = 240 MB

Compression = 30%

Segment size = 1400 bytes

Frame overhead = 60 bytes

Calculation

Compressed size

240×0.70=168 MB

Compressed bytes

168,000,000 bytes

Segments

168,000,000÷1400=120,000

Protocol overhead

120,000×60=7,200,000 bytes

Final transmitted data

168+7.2=175.2 MB

Answer

- Compressed file size = 168 MB

- Segments transmitted = 120,000

- Protocol overhead = 7.2 MB

- Total data transmitted = 175.2 MB

Explanation

The Presentation Layer compresses

and encrypts the data. The Transport Layer divides the data into segments for reliable delivery. The Data Link Layer adds frame headers and trailers for error detection. Finally, the Physical Layer transmits the complete data over the communication medium. Together, these OSI layers provide secure, reliable, and efficient communication.