SIMATS ENGINEERING

ASSESSMENT TOOL 2

NAME: Siva Dakshitha K                                                 REG.NO: 192425395

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0703

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

ANS:

1. Network Design for a 20-Person Startup

Problem Analysis

A startup with approximately 20 employees requires a network that is affordable, reliable, easy to manage, and capable of supporting future growth. Employees may use desktop computers, laptops, printers, VoIP phones, IP cameras, and other network devices.

The network should provide:

- Reliable communication between employees.

- High-speed Internet access.

- File and printer sharing.

- Wireless connectivity.

- Basic network security.

- Easy troubleshooting.

- Future expansion.

Proposed Topology

The recommended topology is Star Topology.

In a star topology, every end device is connected to a central network switch. The switch acts as the central point for communication.

Basic Structure

Internet → Router/Firewall → Managed Switch → End Devices

The end devices include:

- Employee computers

- Laptops

- Network printers

- VoIP phones

- Wireless access points

- IP cameras

Network Components

| Component | Purpose |
| --- | --- |
| 24-port managed switch | Central connection point |
| Router/firewall | Internet, NAT and security |
| Wi-Fi access point | Wireless connectivity |
| Cat6 UTP | Wired connections |
| Network printer | Shared printing |
| UPS | Backup power |
| DHCP server/service | Automatic IP assignment |

A 24-port switch is suitable because approximately 20 employees may require ports while additional ports remain available for printers, access points, phones, and future users.

IP Addressing

Example network:

Network: 192.168.10.0/24
Gateway: 192.168.10.1
DHCP range: 192.168.10.50 – 192.168.10.200

Devices such as printers and network equipment can use reserved or static IP addresses.

Security

The startup can improve security by implementing:

- Strong administrator passwords.

- WPA2/WPA3 for Wi-Fi.

- Firewall rules.

- VLANs for different device categories.

- Guest Wi-Fi isolation.

- Regular firmware updates.

- Access control.

Working

When an employee sends data to another computer, the data first reaches the central switch. The switch determines the destination device and forwards the frame to the appropriate port.

If one workstation cable fails, only that workstation is affected. Other employees continue working normally.

Advantages

- Easy installation.

- Easy troubleshooting.

- Good network performance.

- Simple expansion.

- Failure of one cable affects only one device.

- Centralized management.

- Supports Gigabit Ethernet.

- Suitable for small and medium organizations.

Limitations

The main disadvantage is that the central switch is a critical point. If the switch fails, connected devices lose network connectivity.

This can be addressed by:

- Keeping a spare switch.

- Using redundant switches for critical systems.

- Connecting important devices through backup links.

Testing

The network can be tested using:

- ping for connectivity.

- ipconfig or ip addr for IP configuration.

- tracert/traceroute for path testing.

- Switch management interface for port status.

- Speed tests for network performance.

Conclusion

Star topology is the most appropriate solution for a 20-person startup because it provides simple management, good performance, fault isolation, scalability, and relatively low cost.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

ANS:

2. High-Availability Network Design for a Bank

Problem Analysis

A bank requires extremely reliable networking because banking applications, ATMs, online transactions, databases, security systems, and employee services depend on continuous network availability.

A network failure can result in:

- Transaction delays.

- ATM downtime.

- Loss of customer access.

- Business interruption.

- Financial losses.

- Security and compliance problems.

Therefore, the network must provide redundancy, fault tolerance, security, high bandwidth, and rapid recovery.

Proposed Topology

A Partial Mesh Topology is recommended for the bank's core network.

A full mesh provides a direct connection between every device, but it becomes expensive and difficult to manage as the number of devices increases. Therefore, a partial mesh is more practical.

Network Structure

Example:

Internet ISP 1 → Router 1 → Firewall 1 → Core Switch 1

Internet ISP 2 → Router 2 → Firewall 2 → Core Switch 2

The core switches are connected using multiple redundant links.

Important servers can have redundant network connections.

Network Components

| Component | Purpose |
| --- | --- |
| Dual routers | Redundant Internet/WAN connectivity |
| Dual firewalls | Security and failover |
| Core switches | High-speed network backbone |
| Distribution switches | Connect departments |
| Fiber optic | High-speed backbone |
| Servers | Banking applications and databases |
| UPS | Power protection |
| Backup systems | Disaster recovery |

Routing

Routing protocols such as OSPF or BGP can be used depending on the network architecture.

OSPF can provide dynamic routing inside the organization.

BGP can be used for connections between different autonomous systems, such as multiple Internet service providers.

If one path fails, routing can select an alternative available path.

Security

Bank networks require strong security mechanisms such as:

- Firewalls.

- VLAN segmentation.

- Access Control Lists.

- IDS/IPS.

- Network monitoring.

- Encryption.

- Multi-factor authentication.

- Secure remote access.

- Security logging.

- Regular vulnerability assessments.

Sensitive systems such as databases and payment systems should be separated from ordinary employee networks.

Fiber Connectivity

Fiber optic cables are preferred for the backbone because they provide:

- High bandwidth.

- Long-distance transmission.

- Low electromagnetic interference.

- High reliability.

- Low latency.

Failure Scenario

Suppose Core Switch 1 fails.

Traffic can be redirected through Core Switch 2 using the redundant network path.

Similarly, if one Internet connection fails, the second ISP connection can continue providing connectivity.

This demonstrates fault tolerance.

Testing

The network should be tested using real-world failure scenarios:

- Disconnect one core link.

- Shut down one router.

- Test firewall failover.

- Test Internet connection failure.

- Test server connectivity.

- Monitor routing convergence.

- Verify security rules.

Advantages

- Very high availability.

- Multiple communication paths.

- Fast recovery.

- Reduced downtime.

- High bandwidth.

- Better disaster tolerance.

- Suitable for critical applications.

Limitations

- Higher cost.

- More complex configuration.

- Requires skilled administrators.

- Requires additional hardware and maintenance.

Conclusion

A partial mesh with redundant routers, switches, firewalls, links, and power systems provides the availability, reliability, security, and fault tolerance required by modern banking infrastructure.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8- or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

ANS:

3. Low-Cost Network for a Classroom Lab

Problem Analysis

A small computer laboratory may contain 10–30 computers. Since the budget is limited, the network should provide basic connectivity without expensive enterprise equipment.

The main requirements are:

- Low installation cost.

- Easy configuration.

- Internet access.

- Computer-to-computer communication.

- File and printer sharing.

- Simple maintenance.

Proposed Solution

For a modern Ethernet classroom, a Star topology using an inexpensive unmanaged switch is recommended.

Although traditional bus topology is historically associated with coaxial Ethernet, it is rarely used in modern LANs.

Network Design

Router → Unmanaged Switch → Computers

For example, an 8-port or 16-port switch can connect the computers.

If more computers are required, additional switches can be connected.

Components

| Component | Purpose |
| --- | --- |
| 8/16-port switch | Connects computers |
| Router | Internet access |
| Cat5e/Cat6 cables | Network connections |
| PCs | End devices |
| Printer | Shared resource |
| UPS | Protects network equipment |

IP Configuration

Example:

Network: 192.168.20.0/24
Gateway: 192.168.20.1
Computer 1: 192.168.20.10
Computer 2: 192.168.20.11
Computer 3: 192.168.20.12

DHCP can automatically assign IP addresses.

Working

When Computer 1 sends data to Computer 2, the switch receives the Ethernet frame and forwards it toward the destination port.

The router is used when communication needs to leave the local network, such as accessing the Internet.

Advantages

- Low cost.

- Easy installation.

- Easy troubleshooting.

- Simple configuration.

- Easy to expand.

- Good performance for classroom workloads.

Limitations

- Central switch failure can affect the entire lab.

- Unmanaged switches provide limited monitoring.

- No advanced routing features.

- Security features are limited.

Testing

Students can test the network using:

ping 192.168.20.11

They can also test Internet connectivity using:

ping 8.8.8.8

The teacher can check IP configuration using:

ipconfig

or:

ip addr

Conclusion

A low-cost switched star network is the most practical modern solution for a classroom because it provides simple installation, reliable connectivity, low cost, and easy expansion.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

ANS:

4. Hybrid Network Design for a University Campus

Problem Analysis

A large university may contain thousands of users distributed across many buildings such as:

- Engineering blocks.

- Lecture halls.

- Laboratories.

- Libraries.

- Hostels.

- Administrative offices.

- Research centers.

- Cafeterias.

A single simple topology cannot efficiently support such a large environment.

The network therefore requires:

- High bandwidth.

- Scalability.

- Redundancy.

- Centralized management.

- Wireless access.

- Security.

- Departmental segmentation.

Proposed Solution

A Hybrid Topology is recommended.

It combines different topologies according to the requirements of each part of the campus.

Campus Backbone

Use a fiber-based ring or partial mesh.

Individual Buildings

Use a star topology.

Overall Architecture

Use the traditional:

Core → Distribution → Access

model.

Core Layer

The core layer provides high-speed connectivity between major buildings.

It can use:

- Fiber optic cables.

- Layer 3 switches.

- Redundant links.

- Dynamic routing.

Distribution Layer

Each building can have distribution switches connected to the campus core.

For reliability, important buildings can have two fiber uplinks.

Access Layer

Access switches connect:

- PCs.

- Printers.

- IP phones.

- Wireless access points.

- Cameras.

Wireless Network

Wireless access points can be connected using Power over Ethernet (PoE).

PoE allows both:

- Network data.

- Electrical power

to travel through the same Ethernet cable.

VLAN Design

Example:

| User Group | VLAN |
| --- | --- |
| Students | 10 |
| Faculty | 20 |
| Administration | 30 |
| Laboratory | 40 |
| Guest | 50 |
| CCTV | 60 |

VLANs separate different groups logically and improve security.

Example Architecture

Internet

↓

Campus Firewall

↓

Core Switches

↓

Building Distribution Switches

↓

Floor Access Switches

↓

PCs / Wi-Fi / Printers / IP Phones

Security

Security can be improved using:

- Firewalls.

- VLANs.

- ACLs.

- WPA3.

- Network monitoring.

- IDS/IPS.

- Authentication systems.

- Guest network isolation.

Failure Scenario

If one fiber link between a building and the core fails, the building can use its second redundant link.

Therefore, a single cable failure does not necessarily disconnect the building.

Testing

Testing should include:

- Ping between buildings.

- VLAN connectivity testing.

- Internet connectivity.

- Wi-Fi coverage testing.

- Fiber link failure testing.

- Redundant path testing.

- Bandwidth testing.

- Firewall and ACL testing.

Advantages

- Highly scalable.

- High performance.

- Redundant backbone.

- Easy building expansion.

- Centralized management.

- Strong security.

- Supports wired and wireless users.

Conclusion

A hybrid topology with a redundant fiber backbone, hierarchical architecture, star-based building networks, VLANs, and wireless connectivity is an effective solution for a large university campus.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

ANS:

5. Secure Network Design for Multiple Departments

Problem Analysis

A company may contain departments such as HR, Finance, Sales, Marketing, and IT. All departments need network connectivity, but sensitive departments should not have unrestricted access to each other's systems.

For example:

- HR contains employee information.

- Finance contains financial information.

- IT manages network infrastructure.

- Sales contains customer information.

Therefore, network segmentation is necessary.

Proposed Solution

A hierarchical switched network with VLANs and inter-VLAN routing is recommended.

Network Architecture

Internet

↓

Firewall/Router

↓

Layer 3 Core Switch

↓

Department Access Switches

↓

End Devices

VLAN Design

| Department | VLAN | Network |
| --- | --- | --- |
| HR | 10 | 192.168.10.0/24 |
| Finance | 20 | 192.168.20.0/24 |
| Sales | 30 | 192.168.30.0/24 |
| IT | 40 | 192.168.40.0/24 |
| Guest | 50 | 192.168.50.0/24 |

Each VLAN represents a logically separate network.

Inter-VLAN Routing

A Layer 3 switch can perform routing between VLANs.

For example:

HR VLAN → Layer 3 Switch → Finance VLAN

However, access can be restricted using ACLs.

For example, HR users may be allowed to access a shared HR server but blocked from accessing Finance databases.

Security

The company can implement:

- VLAN segmentation.

- ACLs.

- Firewall rules.

- WPA3 Wi-Fi.

- Strong authentication.

- Port security.

- Network monitoring.

- IDS/IPS.

- Guest network isolation.

IP Addressing

Each department receives a separate subnet.

Example:

HR:

192.168.10.0/24

Finance:

192.168.20.0/24

Sales:

192.168.30.0/24

This makes network management easier.

Advantages

- Better security.

- Reduced broadcast traffic.

- Improved network performance.

- Easier troubleshooting.

- Controlled communication.

- Easy expansion.

- Logical separation of departments.

- Better resource management.

Testing

Testing should include:

Test 1 — Same VLAN

HR Computer 1 → HR Computer 2

Expected result: Communication allowed.

Test 2 — Authorized Inter-VLAN

HR Computer → Approved HR Server

Expected result: Communication allowed.

Test 3 — Restricted Access

Sales Computer → Finance Database

Expected result: Blocked if the security policy prohibits it.

Test 4 — Internet

All authorized departments → Internet

Expected result: Internet access allowed through the firewall.

Conclusion

A VLAN-based hierarchical network provides security, performance, scalability, and centralized management. Inter-VLAN routing allows necessary communication while ACLs and firewalls prevent unauthorized access.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |