SIMATS ENGINEERING

ASSESSMENT TOOL 3

NAME: Siva Dakshitha K                                                     REG.NO: 192425395

COURSE NAME: Computer Networks                     COURSE CODE: CSA0703

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

QUESTIONS: 5								Total Marks:25

Annexure A

Comparative Analysis

- Star topology is generally more expensive than bus topology because it requires a central device such as a switch or hub and more cabling to connect each node individually. In contrast, bus topology uses a single backbone cable, making it low-cost and easier to install. However, in terms of reliability, star topology is superior because failure of one node does not affect others, whereas a failure in the main bus cable can bring down the entire network. Performance is also better in star topology since data transmission is managed through a central device, reducing collisions, while bus topology suffers from performance degradation as more devices are added due to increased data traffic on the shared medium.

ANS:

1. Compare Star and Bus Topology Based on Cost, Reliability and Performance

Introduction

Network topology describes the physical or logical arrangement of devices and communication links in a computer network. Star and bus are two important network topologies. They differ significantly in terms of cost, reliability, performance, maintenance, scalability, and failure handling.

A. Cost Comparison

Bus Topology

Bus topology uses a common backbone cable to which multiple devices are connected. Since a single communication cable is shared by several devices, the amount of cabling required is relatively small.

Traditional bus networks also do not require a central switch.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

The administrator can isolate the problem to the affected network segment.

Therefore:

- Less cable is required.

- Fewer network devices are required.

- Initial installation cost is low.

- Hardware requirements are simple.

Star Topology

In star topology, every device has a separate connection to a central switch.

For example, if 20 computers are connected, each computer requires an individual cable to the switch.

The network therefore requires:

- Central switch.

- Individual cables.

- Additional ports.

- More cable management.

Therefore, the initial installation cost can be higher than a traditional bus network.

Cost Verdict

Bus topology → Lower initial cost

Star topology → Higher initial cost but better long-term value

B. Reliability Comparison

Reliability refers to the ability of the network to continue operating correctly.

Bus Topology

All devices depend on the common backbone.

If the backbone cable is damaged, the entire network or a large portion of it can become unavailable.

This creates a major single point of failure.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Star Topology

Each device has an independent connection to the central switch.

If Computer 5's cable is damaged:

Computer 5 → Disconnected

But:

Computer 1, 2, 3, 4, 6, 7... → Continue working

Therefore, failure is isolated to the affected device.

However, the central switch is still a critical component. If the switch fails, all devices connected to it may lose connectivity.

Reliability Verdict

Star topology > Bus topology

C. Performance Comparison

Bus

All devices share the same communication medium.

When network traffic increases:

- More devices compete for the medium.

- Network performance decreases.

- Traditional shared Ethernet can experience collisions.

- Response time can increase.

Therefore, bus topology is not suitable for high-traffic modern networks.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Star

In a switched star network, each device connects to a switch port.

Modern Ethernet switches can forward frames directly toward the destination port, providing better utilization and performance than traditional shared Ethernet. Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Advantages include:

- Higher throughput.

- Reduced collision domains.

- Better traffic management.

- Dedicated links to end devices.

- Easier bandwidth upgrades.

Performance Verdict

Star topology > Bus topology

D. Scalability

Star topology is easier to expand.

Suppose an organization has a 24-port switch and initially connects 15 computers. Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

There are still 9 ports available.

Additional devices can be connected directly.

In a bus network, adding devices increases the number of devices sharing the same communication medium, which can reduce performance. Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Therefore:

Star → Better scalability

Bus → Limited scalability

E. Maintenance

Star topology makes troubleshooting easier.

If one computer fails, the administrator can check:

- Computer network adapter.

- Ethernet cable.

- Switch port.

- IP configuration.

In a bus topology, a backbone fault can affect many devices and locating the fault can be difficult.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Therefore:

Star → Easy maintenance

Bus → More difficult maintenance

F. Practical Example

Consider a startup with 20 employees.

Bus Network

One backbone cable connects all computers.

If the backbone develops a fault:

Many/all computers → Network unavailable

Star Network

All computers connect to a central switch.

If Computer 8's cable fails:

Computer 8 → Offline

Other 19 computers → Continue operating

Therefore, star topology is more suitable for a modern startup.

Final Comparison

| Parameter | Star | Bus |
| --- | --- | --- |
| Initial cost | Higher | Lower |
| Cabling | More | Less |
| Reliability | High | Low |
| Performance | High | Lower |
| Scalability | Good | Limited |
| Troubleshooting | Easy | Difficult |
| Central device | Required | Not traditionally required |
| Failure impact | Usually one node | Backbone failure can affect network |
| Modern usage | Very common | Mostly obsolete |
| Suitable for | Offices, schools, enterprises | Historical/specialized scenarios |

Conclusion

Although bus topology has a cost advantage, star topology is superior for most modern LAN environments because it provides better performance, reliability, scalability, fault isolation, and easier maintenance. Therefore, the additional cost of star topology is justified in professional networks. Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

- Mesh topology offers very high fault tolerance because every node is connected to multiple other nodes, providing multiple paths for data transmission. If one link fails, data can still be routed through alternate paths without disrupting communication. In contrast, star topology has moderate fault tolerance; while failure of individual nodes does not affect the network, failure of the central hub or switch can cause the entire network to stop functioning. Therefore, mesh topology is more robust and reliable in critical environments compared to star topology.

ANS:

2. Compare Mesh and Star Topology Based on Fault Tolerance

Introduction

Fault tolerance is the ability of a network to continue providing services even when one or more components fail.

Mesh and star topologies provide different levels of fault tolerance.

A. Mesh Topology

In a mesh topology, devices are connected through multiple communication paths.

In a full mesh, every node is directly connected to every other node.

The number of links required for a full mesh is:

L = n(n − 1) / 2

where:

- L = number of links

- n = number of devices

For 6 devices:

L = 6(6 − 1) / 2

L = 15 links

This large number of links creates redundancy.

B. Failure Handling in Mesh

Consider three nodes:

A — B — C

with additional connections between them.

If the direct connection between A and B fails, A may still communicate with B through C.

Therefore, the network can continue operating.

This is the major advantage of mesh topology.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

C. Star Topology

In star topology:

Computer → Central Switch

Every device depends on the central switch for local connectivity.

If one computer's cable fails:

Only that computer is affected.

However, if the central switch fails:

All connected devices may lose connectivity.

Therefore, the switch is a central point of failure.

D. Fault Tolerance Comparison

| Parameter | Mesh | Star |
| --- | --- | --- |
| Alternative paths | Many | Usually few |
| Fault tolerance | Very high | Moderate/High |
| Single point of failure | Low | Central switch |
| Link failure impact | Usually limited | Usually one device |
| Device failure impact | Usually limited | Usually one device |
| Cost | Very high | Moderate |
| Complexity | Very high | Low |
| Maintenance | Difficult | Easy |

E. Real-World Example

Banking

Banks require highly available communication.

Suppose one link between two core network devices fails.

A mesh or partial-mesh design can route traffic through another path.

Therefore, the transaction network can continue operating.

Office

A normal office can use star topology.

If one employee's Ethernet cable fails, only that employee is disconnected.

There is no need for every computer to have multiple direct links.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

F. Full Mesh vs Partial Mesh

A full mesh provides maximum redundancy but can become extremely expensive.

A partial mesh provides redundant connections only where they are most important.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

For example:

Core Switch 1 ↔ Core Switch 2

Core Switch 1 ↔ Core Switch 3

Core Switch 2 ↔ Core Switch 3

while ordinary access devices can use star connections.

This provides a good balance between cost and fault tolerance.

Conclusion

Mesh topology provides greater fault tolerance because multiple paths are available. Star topology is less redundant but is simpler and cheaper. Therefore, mesh is suitable for critical networks, while star is more suitable for ordinary LANs.

- Bus topology is simple and easy to install because it uses a single backbone cable with minimal connections, making it suitable for small networks. On the other hand, mesh topology is highly complex to install as each node must be connected to every other node, requiring a large number of cables and ports. This complexity increases significantly as the number of devices grows, making mesh topology time-consuming and costly to implement compared to the straightforward setup of bus topology.

ANS:

3. Compare Bus and Mesh Topology Based on Installation Complexity

Introduction

Installation complexity depends on the number of cables, network interfaces, devices, configuration requirements, physical space, and maintenance effort.

Bus and mesh represent two extremes.

Bus is simple, while mesh is highly interconnected.

A. Bus Installation

Traditional bus topology uses one main backbone cable.

Devices are attached to the shared medium.

The installation requires:

- Backbone cable.

- Device connections.

- Appropriate termination for traditional coaxial Ethernet.

- Network interface hardware.

The amount of cable is relatively low.

Therefore, installation is simple for a small network.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

B. Mesh Installation

In a full mesh, every device requires a connection to every other device.

The number of connections increases rapidly.

Formula:

L = n(n − 1) / 2

Example: 4 Devices

4 × 3 / 2 = 6 links

Example: 8 Devices

8 × 7 / 2 = 28 links

Example: 10 Devices

10 × 9 / 2 = 45 links

Therefore, adding more devices greatly increases installation complexity.

C. Cabling Comparison

Bus:

One shared backbone

Mesh:

Multiple direct connections

Therefore:

Bus → Less cabling

Mesh → Much more cabling

D. Configuration

Bus networks have relatively simple physical configuration but traditional bus Ethernet requires correct termination and careful cable layout.

Mesh networks require:

- Multiple interfaces.

- Multiple links.

- Routing configuration.

- Redundancy planning.

- Monitoring.

- Failure detection.

Therefore, mesh requires more technical expertise.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

E. Cost

Bus topology has:

- Lower cabling requirements.

- Lower hardware requirements.

- Lower installation cost.

Mesh requires:

- More cables.

- More interfaces/ports.

- More network equipment.

- More installation labor.

- More maintenance.

Therefore:

Mesh → Much higher cost

F. Scalability

Adding one device to a bus requires attaching it to the shared medium.

However, adding devices increases traffic on the shared medium.

In full mesh, adding one new device requires links to all existing nodes.

For example, if a network has 10 nodes and a new 11th node is added, the new node needs 10 additional links in a full mesh.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails. Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

This demonstrates why mesh becomes expensive as the network grows.

Comparison

| Factor | Bus | Mesh |
| --- | --- | --- |
| Installation | Simple | Complex |
| Cabling | Low | Very high |
| Cost | Low | Very high |
| Configuration | Simple | Complex |
| Scalability | Limited | Expensive |
| Fault tolerance | Low | Very high |
| Required expertise | Low | High |
| Maintenance | Moderate | Difficult |

Conclusion

Bus topology is easier and cheaper to install because it uses a shared backbone, while mesh topology requires many connections and more configuration. However, the additional complexity of mesh provides high redundancy and fault tolerance.

Therefore:

Bus = Simple + Cheap

Mesh = Complex + Reliable

- Hybrid topology is highly scalable because it combines two or more different topologies (such as star, mesh, or bus), allowing the network to expand flexibly according to organizational needs. New segments can be added without affecting the existing network structure. In comparison, star topology is also scalable but to a limited extent, as it depends on the capacity of the central device (switch or hub). Once the central device reaches its port limit, expansion requires additional hardware. Thus, hybrid topology provides greater scalability than star topology.

ANS:

4. Compare Hybrid and Star Topology Based on Scalability

Introduction

Scalability is the ability of a network to increase its number of users, devices, buildings, departments, and services without requiring a complete redesign.

Both star and hybrid topologies can be scalable, but hybrid topology provides greater flexibility for large organizations.

A. Star Scalability

In star topology, devices connect to a central switch.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Suppose a company initially has:

15 computers → 24-port switch

As the company grows:

15 → 20 → 24 computers

The remaining ports can be used.

When all ports are occupied, another switch can be added.

Therefore, star topology is reasonably scalable.

B. Limitations of Star

The main limitation is dependence on the central infrastructure.

Expansion may require:

- Additional switches.

- More uplinks.

- Additional rack space.

- More cables.

- More power.

If the network grows significantly, a simple single-switch star design may no longer be sufficient. Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

C. Hybrid Topology

Hybrid topology combines different network topologies.

For example, a university can use:

Campus → Mesh/Ring

Building → Star

Floor → Star

This allows each part of the network to use the most appropriate topology.

D. Example: University

Imagine a university has:

- 5 academic buildings.

- 2 libraries.

- 4 hostels.

- 1 administration block.

A single star network would require all buildings to depend on a central point.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Instead, a hybrid design can use:

Fiber backbone

↓

Core switches

↓

Building distribution switches

↓

Access switches

↓

End devices

This makes expansion easier.

If a new building is constructed, it can be connected to the existing backbone.

E. Flexibility

Hybrid topology allows organizations to choose different designs for different requirements.

For example:

| Area | Topology |
| --- | --- |
| Campus backbone | Mesh/Ring |
| Building | Star |
| Data center | Partial Mesh |
| Office floor | Star |
| Wireless network | Star through AP/switch |

This flexibility is the main advantage of hybrid topology.

Comparison

| Factor | Star | Hybrid |
| --- | --- | --- |
| Scalability | Good | Very high |
| Flexibility | Moderate | Excellent |
| Complexity | Low | High |
| Cost | Moderate | Variable |
| Expansion | Switch-based | Segment-based |
| Suitable size | Small/medium | Medium/large |
| Management | Easy | More complex |

Conclusion

Star topology is scalable for small and medium networks, but hybrid topology is more suitable for large organizations because different topologies can be combined to satisfy different requirements.

Therefore:

Star = Simple scalability

Hybrid = Flexible large-scale scalability

- In terms of maintenance, star topology is relatively easy to manage because issues can be quickly identified and isolated to a specific node or cable. Mesh topology, while highly reliable, is difficult to maintain due to its complex structure and large number of connections, making troubleshooting time-consuming. Bus topology is harder to maintain than star because faults in the backbone cable can disrupt the entire network and are difficult to locate. Hybrid topology has moderate maintenance complexity; while it benefits from flexibility, managing multiple combined topologies requires careful monitoring and skilled administration.

ANS:

5. Comparison of Star, Mesh, Bus and Hybrid Topologies Based on Maintenance

Introduction

Network maintenance includes monitoring network devices, identifying failures, replacing damaged components, checking cables, configuring devices, and ensuring network availability.

Different topologies have different maintenance requirements.

A. Star Topology

Star topology is one of the easiest topologies to maintain.

Each device has its own connection to a central switch.

Suppose Computer 10 stops working.

The administrator can check:

- Computer.

- Network adapter.

- Ethernet cable.

- Switch port.

- IP configuration.

Other computers normally continue functioning.

Maintenance Level:

Easy

B. Mesh Topology

Mesh topology is highly reliable but difficult to maintain.

There may be many:

- Links.

- Interfaces.

- Routing paths.

- Redundant connections.

- Network devices.

Administrators must monitor all these connections.

However, if one link fails, another link may continue carrying traffic.

Maintenance Level:

Complex

Main Trade-off:

More redundancy → More complexity

C. Bus Topology

Bus topology has relatively simple physical structure but troubleshooting can be difficult.

A fault in the shared backbone can affect multiple devices.

The administrator may need to inspect different parts of the backbone to locate the problem.

Traditional bus networks can also be sensitive to cable and termination problems.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Maintenance Level:

Difficult

D. Hybrid Topology

Hybrid networks combine different topologies.

For example:

- Star at access layer.

- Mesh at core.

- Ring backbone.

- VLAN-based departmental networks.

The advantage is flexibility.

The disadvantage is that administrators must understand multiple network technologies.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Maintenance Level:

Moderate to Complex

Overall Comparison

| Parameter | Star | Bus | Mesh | Hybrid |
| --- | --- | --- | --- | --- |
| Troubleshooting | Easy | Difficult | Complex | Moderate/Complex |
| Fault isolation | Excellent | Poor | Excellent | Good |
| Cabling | Moderate | Low | Very high | Variable |
| Maintenance cost | Low/Moderate | Low initially | High | Variable |
| Complexity | Low | Low/Moderate | Very high | High |
| Reliability | High | Low | Very high | High |
| Required expertise | Moderate | Basic | Advanced | Advanced |

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Practical Scenario

Consider a company with 50 employees.

Star

One employee's cable fails.

Result: One employee loses connection.

Maintenance: Replace/check one cable.

Bus

The backbone develops a fault.

Result: Many devices may lose connectivity.

Maintenance: Locate and repair backbone fault.

Mesh

One connection fails.

Result: Traffic can potentially use another path.

Maintenance: Network remains operational, but administrator must identify and repair the failed link.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Hybrid

A building's access switch fails.

Result: Devices connected to that switch may be affected, while other buildings can continue operating.

The administrator can isolate the problem to the affected network segment.

Conclusion

Star topology provides the best balance between simplicity, reliability, and maintenance for most LANs.

Mesh provides the highest redundancy but requires advanced maintenance.

Bus is inexpensive but is difficult to troubleshoot when the shared backbone fails.

Hybrid provides flexibility and scalability but requires skilled administrators because it combines multiple network designs.

Final Ranking for Maintenance

Easiest → Most Complex

Star → Bus → Hybrid → Mesh

The exact order can vary by implementation, but star is generally easiest to troubleshoot, while full mesh is generally the most complex to maintain.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 15% | Demonstrates complete and in-depth understanding of all concepts being compared | Good understanding with minor gaps | Basic understanding; some concepts unclear | Poor understanding of concepts |
| Comparison Depth | 20% | Provides detailed, insightful, and meaningful comparisons across multiple aspects | Adequate comparison with relevant points | Limited comparison; lacks depth | Minimal or incorrect comparison |
| Criteria Selection | 10% | Selects highly relevant and appropriate comparison parameters | Mostly relevant criteria | Some irrelevant or missing criteria | Poor or no criteria selection |
| Analytical Skills | 15% | Strong analysis with logical reasoning and clear justification | Good analysis with some justification | Basic analysis with weak reasoning | No clear analysis |
| Organization & Structure | 10% | Well-organized, clear, and logically structured | Generally organized | Somewhat disorganized | Poorly structured |
| Use of Examples / Evidence | 10% | Uses strong, relevant examples or data to support comparison | Uses some examples | Limited examples | No supporting examples |
| Clarity of Presentation | 10% | Very clear, concise, and easy to understand | Mostly clear | Somewhat unclear | Difficult to understand |
| Conclusion & Insights | 10% | Insightful conclusion with strong justification | Clear conclusion | Basic conclusion | No or weak conclusion |